# Design Notebook

# 2023-02-18 RF Bringup

- Proto RF board bringup

  - SF2312D SAW Filter Datasheet

  - RF board S21 frequency response with 30dB attenuator on output

  - RF board S11 input match (powered off)

  - RF board S11 input match (powered on)

- Proto detector board bringup

  - Input match without power

  - Input match with power

- Decoding ADS-B Messages

- Notes for next PCB Rev

  - ADS8314 has internal coupling cap, can ditch 39pF cap on input to RF detector board.

## 2023-02-19 MCU Board Bringup

- Notes for next time

  - Remember to look for unconnected in the PCBNew DRC! Boy.

    - 2N7002 MOSFET gate was not connected.

    - Comparator output was not connected to Pi Pico.

  - MCP5941U is a DIFFERENT footprint than the MCP6541! I got the MCP6541U, which is pin compatible with the LMV331 but is different than the MCP6541.

  - MCP6541 is too slow to decode ADS-B at 1MHz. The propagation time is 4us! This results in only a partial slew out of the comparator when the input signal has made the full pulse. Not good.

  - Switched comparator to LMV331 (~400ns propagation time, slowish but maybe fast enough). Need to add an output pullup resistor since the output of the LMV331 is open collector.

## 2023-02-20 Decoding ADS-B?

- Replicating ADS-B waveform with arbitrary function generator.

    - Downsampled to 1:20 sample ratio. 312Msps/20 = 15.6Msps