# Design Notebook

# 2023-02-18 RF Bringup

- Proto RF board bringup

  - SF2312D SAW Filter Datasheet

  - RF board S21 frequency response with 30dB attenuator on output

  - RF board S11 input match (powered off)

  - RF board S11 input match (powered on)

- Proto detector board bringup

  - Input match without power

  - Input match with power

- Decoding ADS-B Messages

- Notes for next PCB Rev

  - ADS8314 has internal coupling cap, can ditch 39pF cap on input to RF detector board.

## 2023-02-19 MCU Board Bringup

- Notes for next time

  - Remember to look for unconnected in the PCBNew DRC! Boy.

    - 2N7002 MOSFET gate was not connected.

    - Comparator output was not connected to Pi Pico.

  - MCP5941U is a DIFFERENT footprint than the MCP6541! I got the MCP6541U, which is pin compatible with the LMV331 but is different than the MCP6541.

  - MCP6541 is too slow to decode ADS-B at 1MHz. The propagation time is 4us! This results in only a partial slew out of the comparator when the input signal has made the full pulse. Not good.

  - Switched comparator to LMV331 (~400ns propagation time, slowish but maybe fast enough). Need to add an output pullup resistor since the output of the LMV331 is open collector.

## 2023-02-20 Decoding ADS-B?

- Got a nice ADS-B packet!

  - Downsampled to 1:20 sample ratio to use with arbitrary function generator. 312Msps/20 = 15.6Msps

  - Replicating ADS-B waveform from comparator output with arbitrary function generator.

- CANRP2040 project has really good examples and notes about using RP2040 PIO for decoding stuff at 1MHz.

- RP2040 Datasheet

- C++ PIO Tutorial

- Bringing up PIO capture program.

  - Testing with 200 cycle “blink” pulses, PIO clock set to 16MHz.

    - 200*80kHz = 16MHz. Not bad!

- Got into some really weird error loops with the debugger when trying to debug PIO. Some troubleshooting tips that may or may not make a difference:

  - Move the SWD cable away from the RF boards / put the cable below the ground planes.

  - Power cycle by turning off the power supply for a full 10 seconds, then reset the board with the debugger and reconnect.

  - Wait until the debugger is done thinking before clicking any buttons.

  - Disable adaptive clocking, set it to something low like 4000kHz (this is not done by default)!

  - This is it!! Launch vs. attach! Since I am running an external debugger session, I need to attach to it instead of launching a new one. Now all the issues make sense (things working the first time but not the second, all of the breakpoints being set over each other, etc).

## 2023-02-24 More Decoding ADS-B

- Successful preamble detection on a recorded message!

  - Used PIO to detect a preamble and set a GPIO HI right after a valid one was found.